Specifications:

# GENERAL

Crew . . . . . . . . . . . . . . . . . . 2

Passengers . . . . . . . Up to 180 (dense)

Typical Two-class . . . . . . 150

# ENGINES

2 CFMI CFM56-5 or

2 IAE V2500 with up to 26,500lb thrust each

# AVIONICS

Two crew member design

Two primary flight displays (PFD)

Two navigation displays (ND)

Electronic Centralised Aircraft Monitor (ECAM)

Two Multipurpose Control and Display Units (MCDU)

# DIMENSIONS

Span 111ft 10in

Length 123ft 23in

Height 38ft 7in

Fuselage width 12ft 11in

wing sweepback 24.96 deg. @ quarter chord

wing area 1,317.5sq ft

Range 2,700 - 2,900nm

# WEIGHTS

MTOW 162,000lb

MLW 142,200lb

MZFW 134,500lb

operating weight empty 92,100lb

max fuel capac. 6,300USg

Engine thrust 2x 25,000lb - 26,500lb

## PERFORMANCE

# RANGE

Range (with 150 passengers):

2,900nm

# SPEEDS

Max operating speed 350kts 0.82mach

Max cruise speed 487kts @

28,000ft 454kts @37,000ft

# FIELD PERFORMANCE

TO field length 5,630ft

Landing field length 4,750ft

## Data for flying…

CLIMB

V1,Vr,V2 Take-Off speeds @ 150,000#, 145,000#, and 140,000#

(ISA Conditions) / Configure 1: Flaps+1 (Flaps 2 is never used for T/O @ NWA)

T/O weight V1 / Vr / V2 Flaps

150,000# 148 /148 /148 1+F

145,000# 146 /146 /146 "

140,000# 143 /143 /143 "

ROC(FPM) and speed (Kts) (Below 10,000')

aprox. 3800 FPM at 250 Kts

ROC(FPM) and climb speed (Kts/Mach) (Above 10,000' to Cruise FL)

aprox. 2200 FPM at 310 Kts (from 10,000' - 20,000')

aprox. 1800 FPM at 310 Kts (from 20,000' - 26,000')

aprox. 1200 FPM at .78M (from 26,000' - 35,000')

# CRUISE

Typical Cruise Speed @ 135,000# (IAS/Mach cruise) FL350

Standard Cruise Speed at NWA is .78M at all weights from FL310 thru FL390

LRC (Long Range Cruise) @ 135,000#

FL390 - FL370 = .79M

FL350 = .78M

FL330 = .77M

FL310 = .74M

FL290 = .71M

MCC (Minimum Cost Cruise) Varies with winds, fuel costs, crew costs,

altitude, and weight. Not calculated at this time.

DESCENT

ROD(FPM) and speed (Mach/Kts) (From Cruise FL to 10,000')

Hold .78M 'til 290 Kts. Idle power - aprox. 2500 FPM

ROD(FPM) and speed (Kts) (Below 10,000' to FAF)

ROD (rate of descent) varies with ATC requirements, but is between 1000

and 3000 FPM. In a "Perfect World" - 250 Kt idle descent, then slow and

configure on speed so as to not add power until on glide slope.

Vref Approach Bug speeds @ 130,000#, 135,000#, and 140,000#

Flaps FULL

LNDG weight Vref Flaps

130,000# 129 FULL

135,000# 132 "

140,000# 134 "

AIRPORT OPERATIONS:

General

Runway slope (mean) +/- 2%

Runway altitude 8000 ft

Wind:

TO and LDG

X wind: max demonstrated for takeoff 29 Kt

X wind: max demonstrated for landing 33 Kt

X wind: max demonstrated gust 38 Kt

max tail wind 10 Kt

*Max crosswind values have been demonstrated with flight controls in normal law as well as in direct law with and without yaw damper.

Door operations (cabin, cargo) 65 kt

SPEED LIMITATIONS:

VMCA 105 Kt

VMCG conf 1 + F 107 Kt

conf 2 105 Kt

conf 3 105 Kt

MAX FLAPS, SLATS speeds:

| Position | Slats | Flaps | Speed | Remarks |
| --- | --- | --- | --- | --- |
| 0 | 18 | 0 | 230 | Int Appr |
| 1 | 18 | 10 | 215 | T.O. |
| 2 | 22 | 15 | 200 | T.O./Appr |
| 3 | 22 | 20 | 185 | T.O./Appr/Ldg |
| 4/Full | 27 | 35 | 177 | Ldg |

GEAR DOWN SPEEDS:

Max speed with landing gear extended (VLE) 280 Kt/ M.67

Max speed at which the landing gear may be extended (VLO extension) 250 Kt

retracted (VLO retraction) 220 Kt

MAX TYRE SPEED:

Ground speed 195 kt

POWER PLANT:

Thrust Setting/EGT Limits:

| Operating Condition | Time Limit | EGT |
| --- | --- | --- |
| T.O. and GA | 5 Min | 890 C |
| MCT | Unlimited | 855 C |
| CL | Unlimited | 855 C |
| Starting |  | 725 C |

OIL:

Max continuous temperature 140 C

Max transient temperature (15 min) 155 C

Minimum starting temperature -40 C

RPM:

N1 max 102%

N2 max 105%

REVERSE THRUST:

Selection of REV thrust in flight is prohibited

Backing of aircraft with reverse thrust is not permitted. Maximum reverse should not be used

below 70 Kts (idle reverse is allowed down to a/c stop).